# N52922

# Standard Procedures

1981 Cessna 172P - SN 17274635

AIRCRAFT ACCEPTANCE

Parking Brake	SET

Tachometer Time	LOGGED

AROW	CHECKED

PREFLIGHT INSPECTION

1. Cabin

Control Lock	REMOVED

Pitot Cover	REMOVED

Master Switch	OFF

Magnetos	OFF

All Avionics	OFF

Fuel Selector Valve	BOTH

2. Empennage

Baggage Door	LOCKED

Static Ports	CHECKED

Horizontal Stab/ Elevator	CHECKED

Vertical Stab/Rudder	CHECKED

Antennas	CHECKED

Tail Tie Down	Removed

3. Right Wing

Flap and Aileron	CHECKED

Wing Tip/Leading Edge	CHECKED

Strut/Tie Down	Removed

Tire and Brake	CHECKED

Fuel Drain	CHECKED

Fuel Level/Cap	CHECKED/SECURED

4. Nose

Engine Oil (6 qt LFC min )	CHECKED

Tire and Strut	CHECKED

Fuel Drain	CHECKED

Air Filter	CLEAR

Alternator Belt	CHECKED

Propeller and Spinner	CHECKED

Landing Light	CHECKED

Static Source	CHECKED

Windshield	CHECKED

5. Left Wing

Pitot Tube	CHECKED

Fuel Vent	CHECKED

Stall Warning	CHECKED

Strut/Tie Down	Removed

Wing Tip/Leading Edge	CHECKED

Flap and Aileron	CHECKED

Tire and Brake	CHECKED

Fuel Drain	CHECKED

Fuel Level/Cap	CHECKED/SECURED

### STARTING ENGINE

Personal (Optional) Electronics	ON & SET

Passenger Briefing	COMPLETE

Seat and Seatbelts	ADJUST & LOCK

Parking Brakes	SET

Circuit Breakers	Checked

Avionics Switch	OFF

Fuel Selector Valve	Both

Mixture	Full Rich

Throttle	IN ½ inch

Carburetor Heat	OFF

Fuel Prime	TWICE SLOWLY ONLY IF COLD

Master	ON

Foot Brakes	ON

Prop	CLEAR

Ignition Switch	START

Throttle (800-1000 RPM)	SET

Oil Pressure	CHECKED

Flaps	OPERATE / THEN UP

Fuel Quantity	CHECK BOTH GAUGES

Radios	ON & SET

Intercom & Headsets	ON & TEST

Weather Frequency	MONITOR & COPY

Altimeter	SET

Transponder Code	SET / ON

Heading/Compass	ALIGN

Lights	AS NEEDED

Taxi Area	CONFIRM CLEAR

TAXI

Radio	ANNOUNCE INTENTIONS

Brakes	RELEASE, THEN CHECK

BEFORE TAKEOFF RUN-UP

Parking Brake	SET

Seats & Seatbelts	CHECK

Cabin Doors/Windows	CLOSED & LOCKED

Flight Controls	FREE & CORRECT

Flight & Engine Instruments	CHECKED

Mixture	RICH

Trim	Takeoff

Oil Temperature	GREEN

Throttle	1700 RPM

Suction (Vacuum)	4.6-5.4

Magnetos	CHECKED (150 max/50 diff)

Carb Heat	On, Check for Slight Drop, Off

Ammeter	CHECKED

Throttle Idle	CHECKED

Throttle	800-1000 RPM

Throttle Friction	SET

Radios Frequencies (both)	CHECK

Left & Right Fuel Gauges	CHECK

Abort Plan	GOT ONE?

AFTER CLEARED FOR RUNWAY

Trim	Takeoff

Lights and Strobe	AS REQUIRED

Transponder	CHECK CODE/ON

Mixture	As Required (Usually Full Rich)

Carburetor Heat	OFF

TAKEOFF

Flaps (0°-10° As required)	SET

Throttle	FULL

Rotate	57 Knots

CLIMB

Best Rate of Climb (Vy)	76 Knots

Best Angle of Climb (Vx)	62 Knots

Throttle	USUALLY FULL

Mixture	USUALLY FULL

(adjust for max RPM above 3000')

Engine Instruments	MONITOR

CRUISE

Lights & Strobe	AS REQUIRED

Power	AS REQUIRED

Mixture	RICH OF PEAK

Fuel Selector	USUALLY BOTH

Engine Instruments	MONITOR

## VFR Approach Speeds

| Downwind | 2000 RPM Carb Heat | 90 K |
| --- | --- | --- |
| Numbers (10°) | 1700 RPM | 80 K |
| Base (20°) | 1600 RPM | 70 K |
| Final (30°) | 1500 RPM | 65 K |

## Suggested IFR Approach Settings (adjust as needed)

| ILS | 1900 RPM | 500 FPM |
| --- | --- | --- |
| Localizer | 1700 RPM | 700 FPM |
| VOR | 1700 RPM | 700 FPM |

V Speeds

| Normal Climb | 75-85 K |  |
| --- | --- | --- |
| VX (Angle) | 62 K |  |
| VY (Rate) | 76 K |  |
| Best Glide | 70/65 K | Flaps N/Y |
| VA | 105/95/85 K |  |
| VFE | 110/85 K | Flaps N/10/20+ |
| VNE | 158 K |  |
| VNO | 127 K |  |

DESCENT

Power	AS REQUIRED

Mixture	RICH

Fuel Selector Valve	BOTH

Lights & Strobes	ON

Carburetor Heat	ON BELOW GREEN

BEFORE LANDING

Seats & Seatbelts	CHECKED

Fuel Selector	BOTH

Mixture	USUALLY RICH

Carburetor Heat	ON

### AFTER LANDING

- After Clearing The Runway-

Lights	AS REQUIRED

Strobes	OFF

Mixture	LEAN SLIGHTLY

Carburetor Heat	OFF

Flaps	UP

SHUTDOWN, PARK & SECURE

Avionics	OFF

Throttle	IDLE

Mixture	CUT OFF

Ignition Switch	OFF

Master Switch	OFF

Interior/Exterior Lights	OFF

Control Lock	INSTALL

Hobbs time	RECORD IN BOOK

Personal electronics & headsets 	SECURE

Seatbelts & shoulder harnesses	SECURE

Control lock	INSTALL	in

Pitot Cover	INSTALL

Foam tail plug	INSTALL

Tail tie-down	INSTALL

Wing tie-downs	INSTALL

Bug removal	Leading wing edges, struts

Bug removal	Tail leading edge, wheels, nose

Windshield	CLEAN ONLY AS INSTRUCTED

Doors, baggage door	LOCK

Nose plugs	INSTALL

The aircraft is owned and operated by the members of Northwest Florida Flyers, Inc, and is based in Marianna, Florida (KMAI).

http://nwflyers.net/

This normal operations checklist was edited by Ross Statham on May 14, 2022 based from an online Cessna 172P POH, his own previous Cessna 172M checklist and from flying this particular aircraft.

This checklist is a work in progress.  See an error, or have a suggestion for improvement?  Please email Ross at stathamross@gmail.com or call him at 770-329-5400 and thanks!