# N9539H

# Standard Procedures

1981 Cessna 182R - SN 18267961

AIRCRAFT ACCEPTANCE

Parking Brake	SET

Tachometer & Hobbs Time	LOGGED

PREFLIGHT INSPECTION

1. Cabin

Control Lock	REMOVED

Master Switch	OFF

Magnetos	OFF

Avionics Master	OFF

2. Empennage

Baggage Door	LOCKED

Static Ports	CHECKED

Horizontal Stab/ Elevator	CHECKED

Vertical Stab/Rudder	CHECKED

Antennas	CHECKED

Tail Tie Down	SET BY LOCKBOX

3. Right Wing

Flap and Aileron	CHECKED

Wing Tip/Leading Edge	CHECKED

Strut/Tie Down	Removed

Tire and Brake	CHECKED

Fuel Drain	CHECKED

Fuel Level/Cap	CHECKED/SECURED

4. Nose

Engine Oil 	CHECKED

Tire and Strut	CHECKED

Fuel Drain	CHECKED

Air Filter	CLEAR

Alternator Belt	CHECKED

Propeller and Spinner	CHECKED

Landing & Taxi Lights	CHECKED

Transponder Antenna	CLEAR OF OIL

Static Source	CHECKED

Windshield	CHECKED

Front wheel chocks	SET BY LOCKBOX

5. Left Wing

Pitot Tube	REMOVE COVER, CHECK

Fuel Vent	CHECKED

Stall Warning	CHECKED

Strut/Tie Down	Removed

Wing Tip/Leading Edge	CHECKED

Flap and Aileron	CHECKED

Tire and Brake	CHECKED

Fuel Drain	CHECKED

Fuel Level/Cap	CHECKED/SECURED

### STARTING ENGINE

Passenger Briefing	COMPLETE

Seat and Seatbelts	ADJUST & LOCK

Parking Brakes	SET

Circuit Breakers	Checked

Fuel Selector Valve	BOTH

Cowl Flaps	OPEN

Mixture	FULL RICH

Propeller	FULL IN

Throttle	OPEN ½ INCH

Carburetor Heat	OFF

Fuel Prime	TWICE SLOWLY IF COLD

Master	ON

Foot Brakes	ON

Prop	CLEAR

Ignition Switch	START

Throttle (800-1000 RPM)	SET

Oil Pressure	CHECKED

Time	RECORD

Mixture	LEAN AS NEEDED

Flaps	OPERATE / UP

Avionics Master	ON

Radios	ON & SET

GPS Unit	ON & FLIGHT PLAN

Intercom & Headsets	ON & TEST

Weather Frequency	MONITOR & COPY

Altimeter	SET

Transponder	SET CODE

Heading/Compass	ALIGN

Lights	USUALLY ON

Taxi Area	CLEAR

Brakes	RELEASE & TAP TEST

TAXI

Brakes	CHECK

Turn Instruments	CHECK

BEFORE TAKEOFF RUN-UP

Parking Brake	SET

Seats & Seatbelts	CHECK

Flight Controls	FREE & CORRECT

Flight & Engine Instruments	CHECKED

Mixture	RICH

Fuel Selector	BOTH

Trim	TAKEOFF

Oil Temperature	GREEN

Throttle	1700 RPM

Suction (Vacuum)	4.6-5.4

Magnetos	CHECKED (150 max/50 diff)

Carb Heat	On, Check for Slight Drop, Off

Propeller	CYCLE 3 TIMES

Ammeter	CHECKED

Throttle Idle	CHECKED

Throttle	800-1000 RPM

Throttle Friction	SET

Radios/Nav/GPS	Checked

Left & Right Fuel Gauges	Checked

Abort Plan	Complete

AFTER CLEARED ONTO RWY

Cabin Doors/Windows	CLOSED & LOCKED

Lights	ON

Transponder	CHECK CODE & ON

Mixture	As Required (Usually Rich)

Carburetor Heat	OFF

Flaps	As Required

TAKEOFF

Flaps (0°-10°)	SET

Throttle	FULL

Rotate	59 K

CLIMB

Throttle & Propeller	23” MP, 2300 RPM

Mixture	ADJUST FOR MAX RPM

Engine Instruments	CHECKED

CRUISE

Lights	AS REQUIRED

Manifold & RPM	AS NEEDED

Mixture	ADJUST FOR EGT, FUEL FLOW

Flaps	CHECK “UP”

Cowl Flaps	Adjust (usually CLOSED)

Fuel Selector Valve	As Needed

Engine Instruments	CHECKED

DESCENT

Carburetor Heat	ON

Power	AS REQUIRED

Mixture	RICH

Propeller	FULL IN

Fuel Selector Valve	BOTH

Landing & Taxi Lights	ON

Cowl Flaps	USUALLY CLOSED-AS NEEDED

## IFR Approach Settings

| ILS | ~1900 RPM | 500 FPM |
| --- | --- | --- |
| Localizer | ~1700 RPM | 700 FPM |
| GPS | Adjust as required |  |
| VOR | ~1700 RPM | 700 FPM |

## VFR Approach Speeds

| Downwind | 90 Knots |
| --- | --- |
| Base | 80 Knots |
| Final | 70 Knots |
| Threshold | 60 Knots |

V Speeds

| Rotate | 50 K |
| --- | --- |
| VX (Angle) | 59 K |
| VY (Rate) | 81 K |
| Best Glide | 61 K (2000 lb) 77 K (3100 lb) |
| Vs | 40 K |
| Vs 1 | 50 K |
| Vfe 10° | 140 K |
| Vfe 20-40° | 95 K |
| Vno | 143 K |
| Vne | 179 K |
| X Wind | NTE 15 K |
| V turbulence | 102 K (2000 lb) 111 K (3100 lb) |

BEFORE LANDING

Landing & Taxi Lights	ON

Seats & Seatbelts	CHECKED

Fuel Selector	BOTH

Mixture	RICH

Propeller	FULL IN

Carburetor Heat	ON

Autopilot	OFF

Flaps	AS REQUIRED

### AFTER LANDING

Lights	AS REQUIRED

Mixture	LEAN AS NEEDED

Carburetor Heat	OFF

Flaps	UP

Cowl Flaps	OPEN

SHUTDOWN, PARK & SECURE

Parking Brake	USUALLY ON

Avionics	OFF

Throttle	IDLE

Mixture	CUT OFF

Ignition Switch	OFF

Master Switch	OFF

Interior/Exterior Lights	OFF

Control Lock	INSTALL

Hobbs time	RECORD IN BOOK

Personal electronics & headsets 	SECURE

Seatbelts & shoulder harnesses	SECURE

Control lock	INSTALL	in

Pitot Cover	INSTALL

Tail tie-down	INSTALL

Wing tie-downs	INSTALL

Nose gear chocks	INSTALL

Bug removal	Leading wing edges, struts

Bug removal	Tail leading edge, wheels, nose

Windshield	CLEAN ONLY AS INSTRUCTED

Doors, baggage door	LOCK

Nose plugs	INSTALL

Propeller position	VISUALLY AS A “Y”

Cleaning supplies	SECURE IN BOX

MAXIMUM CROSS WIND COMPONENT:

Not to exceed 15 knots

Corrections and suggestions to this list should be sent to Ross Statham

770.329.5400

stathamross@gmail.com

May 19, 2022 Revision